Copyright 1999, The American Institute of Architects (AIA)

SECTION 10190 - CUBICLE CURTAIN TRACK SYSTEMS AND CURTAINS

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

      - SUMMARY

        - This Section includes the following:

Adjust list below to suit Project.

          - Curtain tracks and curtain carriers.

          - Cubicle curtains.

        - Related Sections include the following:

List below only products and construction that the reader might expect to find in this Section but are specified elsewhere.

          - Division 6 Section "Rough Carpentry."

          - Division 9 Section "Gypsum Board Assemblies, Acoustical Panel Ceilings, Acoustical Tile Ceilings."

If alternates, allowances, or unit prices apply to work of this Section, insert brief paragraphs here to alert Contractor and reference appropriate Division 1 Sections for specific details.

      - PERFORMANCE REQUIREMENTS

        - Curtains:  Provide curtain fabrics with the following characteristics:

Retain subparagraph below or revise to suit laundering practices of facility.

          - Fabrics are launderable to a temperature of not less than 160 deg F (71 deg C).

Revise subparagraphs below if required by authorities having jurisdiction.

          - Fabrics are flame resistant and are identical to those that have passed NFPA 701 when tested by a testing and inspecting agency acceptable to authorities having jurisdiction.

            - Identify curtains with size.

      - SUBMITTALS

        - Product Data:  Submit copies of manufacturer’s detailed technical data for materials, fabrication and installation of cubicle curtain tracks and curtains specified herein.  Include catalog cuts of fittings, anchors, fastenings and accessories.

        - Shop Drawings:  Show layout and types of cubicles, sizes of curtains, number of carriers, and conditions requiring accessories.

Retain paragraph below if Drawings do not include detailed reflected ceiling plans or if Project involves unusual coordination requirements.

Delete paragraph above if colors and other characteristics are preselected and specified or scheduled.  Retain paragraph and subparagraphs below with or without above.

        - Samples for Verification:  Full-size units of each type of the following products:

          - Curtain Fabric:  12-inch square swatch or larger Sample as required to show complete pattern repeat, from dye lot used for the Work, with specified treatments applied.  Mark top and face of material.

          - Mesh Top:  Not less than 4 inches square.

          - Curtain Track:  Not less than 4 inches long.

          - Curtain Carrier:  Full-size unit.

        - Cubicle Schedule:  Use same room designations as indicated on Drawings.

        - Product Certificates:  Signed by manufacturers of tracks and curtains certifying that products furnished comply with requirements.

        - Maintenance Data:  For tracks and curtains to include in maintenance manuals specified in Division 1.

      - PROJECT CONDITIONS

        - Environmental Limitations:  Do not install cubicles until spaces are enclosed and weatherproof, wet work in spaces is complete and dry, work above ceilings is complete, and ambient temperature and humidity conditions are maintained at the levels indicated for Project when occupied for its intended use.

        - Field Measurements:  Where cubicles are indicated to fit to other construction, verify dimensions of other construction by field measurements before fabrication and indicate measurements on Shop Drawings.  Coordinate fabrication schedule with construction progress to avoid delaying the Work.

- PRODUCTS

      - MANUFACTURERS

Retain one of four paragraphs below that applies to how products are specified in this Article.  See Division 1 Section "Product Requirements" and Editing Instruction No. 1 in the Evaluations in this Section for cautions about naming products and manufacturers and for advice on which paragraph to retain.

If products and manufacturers are named, retain above for nonproprietary or below for semiproprietary Specification.

If only manufacturers are named, retain above for nonproprietary or below for semiproprietary Specification.  Refer to Division 1 Section "Product Requirements."

        - Manufacturers:  Subject to compliance with requirements, provide products by the following:

          - A. R. Nelson,  3555 Scarlet Oak Blvd., St. Louis,  MO,  63122,  800-377-6625

      - CURTAIN TRACKS

        - Extruded-Aluminum Track No. 1200CT:  1-3/8 inches wide by 3/4 inch high aluminum track.

        - Extruded-Aluminum Suspended Track No. 1100CT:  1-7/16 inches wide by 15/16 inch high aluminum track.

See Evaluations for capability of products to comply with minimum wall thickness in paragraph above and radius in subparagraph below.

          - Curved Track:  Factory fabricated 12-inch radius bends.

          - Finish:  Satin anodized or White powder coat.

        - Track Accessories:  End caps, connectors, end gates, coupling and joining sleeves, wall brackets, ceiling flanges, and other accessories.

          - Suspended Track Support:  Not less than 1-inch OD aluminum tube.

          - End Stop:  Removable.

Delete both subparagraphs below if not required.  Units are only available from Creative Healthcare Products.

Select one of first three paragraphs below.  Breakaway carriers are for use in psychiatric or detention applications; type below is only available from Creative Healthcare Products.  Coordinate type of curtain carrier with size of track above and with products of selected manufacturers; smaller tracks typically use glides instead of rollers.

        - No. 12 Curtain Carriers:  Two nylon rollers, nylon axle and tangle free nylon swivel stem with chrome-plated steel hook.

        - No. 11 Curtain Carriers:  One-piece nylon glide, tangle free nylon swivel with chrome-plated steel hook.

        - No. 20 Breakaway Carrier:  Three-piece nylon wheels, body and hook (recommended for use in No. 1200CT Cubicle Track).

      - CURTAINS

Retain one of first three paragraphs and associated subparagraphs below.  First paragraph is for cubicle manufacturers' standard curtain fabric; second paragraph is for cubicle manufacturers' standard curtain fabric where specific curtain products are named; third paragraph is for C.O.M. curtain fabric.

        - Curtain Fabric:  Cubicle manufacturer's standard, as follows:

          - Fiber Content:  100 percent polyester, inherently and permanently flame resistant.

          - Products:  Subject to compliance with requirements, provide the following:

            - Nelson Cubicle Curtains

Repeat both subparagraphs below for each product named.

              - Pattern:  As selected by Architect from manufacturer's full range of available fabrics.

              - Color:  As selected by Architect from manufacturer's full range of available colors.

Delete first paragraph below if not required.  No. 50 mesh is an open weave with openings of approximately 1/2 inch (13 mm); No. 40 mesh is an open weave with openings of approximately 1/4 inch (6 mm); No. 42 mesh is closely woven with very small openings.  Verify acceptability with authorities having jurisdiction.

        - Mesh Top:  Anti-microbial white nylon mesh with 1/2” diagonal openings.

        - Curtain Grommets:  Two-piece, rolled-edge, rustproof, nickel-plated brass; spaced not more than 6 inches o.c.; machined into top hem.

Retain one of two paragraphs below.

      - CURTAIN FABRICATION

        - Fabricate curtains to comply with the following requirements:

          - Width:  Equal to track length from which curtain is hung plus 10 percent added fullness, but not less than 12 inches added fullness.

Retain one of three subparagraphs below.

          - Length:  Equal to floor-to-ceiling height, with 20-inch mesh top, and minus distance above finished floor at bottom as follows:

Retain applicable subparagraphs below with "Length" Subparagraph selected above.

            - Cubicle Curtains:  14 inches.

See Evaluations for curtain fabrication details of listed manufacturers in five subparagraphs below.

          - Top Hem:  Not less than 1 inch and not more than 1-1/2 inches wide, triple thickness, reinforced with integral web, and double lock stitched.

Retain subparagraph above or below, or both if more than one type is required.

          - Mesh Top:  Top hem not less than 1 inch and not more than 1-1/2 inches wide, triple thickness, reinforced with integral web, and double lock stitched.  Double lock stitch bottom of mesh directly to 1/2-inch triple thickness, top hem of curtain fabric.

          - Bottom Hem:  1 inch double thickness and single lock stitched.

          - Side Hems:  Not less than 1/2 inch and not more than 1-1/4 inches wide, with double turned edges, and single lock stitched.

          - Vertical Seams:  Not less than 1/2 inch wide, double turned and double stitched.

- EXECUTION

      - INSTALLATION

Revise option in paragraph below if track is available in longer lengths.

        - General:  Install tracks level and plumb, according to manufacturer's written instructions.  Provide track fabricated from one continuous length up to 16 feet.

Select one of four options in first subparagraph below.  Suspended tracks are appropriate for ceiling heights over 108 inches (2743 mm).

          - Curtain Track Mounting.  Select One:    Surface     Suspended      Recessed.

Retain one of two paragraphs and associated subparagraphs below.  Retain applicable mountings or delete and show on Drawings.

        - Surface Track Mounting:  Fasten surface-mounted tracks at intervals of not less than 24 inches.  Fasten support at each splice and tangent point of each corner.  Center fasteners in track to ensure unencumbered carrier operation.  Attach track to ceiling as follows:

          - Mechanically fasten directly to bottom of concrete deck with anchor screws.

          - Mechanically fasten directly to finish ceiling with toggle bolts.

          - Mechanically fasten to suspended ceiling grid with screws.

ADC Hospital Equipment, Creative Healthcare Products, and Pryor Products are the only listed manufacturers that provide clips, in paragraph below, for track attachment to suspended ceiling grid.

        - Suspended Track Mounting:  Install track with suspended supports at intervals of not more than 84 inches.  Fasten support at each splice and tangent point of each corner.  Secure ends of track to wall with flanged fittings or brackets.

        - Track Accessories:  Install end caps, connectors, end gates, coupling and joining sleeves, and other accessories as required for a secure and operational installation.

Delete both subparagraphs below if not required.  Coordinate with accessories selected in Part 2.

        - Curtain Carriers:  Provide curtain carriers adequate for 6-inch spacing along the full length of the curtain.

        - Curtains:  Hang curtains on each curtain track.

      - DEMONSTRATION

Retain this Article if safety loading unit is required.

        - Train Owner's maintenance personnel to adjust, operate, and maintain safety- loading units.

          - Train Owner's maintenance personnel on procedures and schedules for changing curtains and     maintaining cubicles.

      - PROTECTION

Delete this Article if no recessed tracks.

        - Protect installed recessed track openings with non-residue adhesive tape to prevent debris from ceiling finishing operations from impeding carrier operation.

- END OF SECTION 10190